修改后的slam参数文件：

slam_toolbox:

ros__parameters:

# Plugin params

solver_plugin: solver_plugins::CeresSolver

ceres_linear_solver: SPARSE_NORMAL_CHOLESKY

ceres_preconditioner: SCHUR_JACOBI

ceres_trust_strategy: LEVENBERG_MARQUARDT

ceres_dogleg_type: TRADITIONAL_DOGLEG

ceres_loss_function: None

# ROS Parameters

odom_frame: odom

map_frame: map

base_frame: base_link

scan_topic: /scan

use_map_saver: true

mode: mapping #localization

# if you'd like to immediately start continuing a map at a given pose

# or at the dock, but they are mutually exclusive, if pose is given

# will use pose

#map_file_name: test_steve

# map_start_pose: [0.0, 0.0, 0.0]

#map_start_at_dock: true

debug_logging: false

throttle_scans: 1

transform_publish_period: 0.02 #if 0 never publishes odometry

map_update_interval: 5.0

resolution: 0.05

min_laser_range: 0.0 #for rastering images

max_laser_range: 20.0 #for rastering images

minimum_time_interval: 0.5

transform_timeout: 0.2

tf_buffer_duration: 30.

stack_size_to_use: 40000000 #// program needs a larger stack size to serialize large maps

enable_interactive_mode: true

# General Parameters

use_scan_matching: true

use_scan_barycenter: true

minimum_travel_distance: 0.5

minimum_travel_heading: 0.5

scan_buffer_size: 10

scan_buffer_maximum_scan_distance: 10.0

link_match_minimum_response_fine: 0.1

link_scan_maximum_distance: 1.5

loop_search_maximum_distance: 3.0

do_loop_closing: true

loop_match_minimum_chain_size: 10

loop_match_maximum_variance_coarse: 3.0

loop_match_minimum_response_coarse: 0.35

loop_match_minimum_response_fine: 0.45

# Correlation Parameters - Correlation Parameters

correlation_search_space_dimension: 0.5

correlation_search_space_resolution: 0.01

correlation_search_space_smear_deviation: 0.1

# Correlation Parameters - Loop Closure Parameters

loop_search_space_dimension: 8.0

loop_search_space_resolution: 0.05

loop_search_space_smear_deviation: 0.03

# Scan Matcher Parameters

distance_variance_penalty: 0.5

angle_variance_penalty: 1.0

fine_search_angle_offset: 0.00349

coarse_search_angle_offset: 0.349

coarse_angle_resolution: 0.0349

minimum_angle_penalty: 0.9

minimum_distance_penalty: 0.5

use_response_expansion: true

min_pass_through: 2

建图运行截图：

遇到的问题与解决方法：

问题1：TF 坐标关系错误

问题描述

初始运行 SLAM 时，机器人激光数据无法正确转换，导致地图无法正常生成。

检查 TF：

ros2 run tf2_tools view_frames

发现坐标关系需要调整。

解决方法

修改 slam_toolbox 参数：

map_frame: map

odom_frame: odom

base_frame: base_link

确保 TF 结构：

map

|

odom

|

base_link

|

lidar_link

问题2：地图出现重影和变形

问题描述

建图过程中出现：

- 墙体重复

- 地图拉伸

- 环境结构错位

原因分析

SLAM 建图依赖：

激光数据(scan)

+

机器人位姿(odom)

+

TF转换

如果：

- odom 不准确

- 时间不同步

- TF错误

会导致激光点被放置到错误位置。

解决方法

检查：

激光数据：

ros2 topic echo /scan --once

确认：

frame_id: lidar_link

检查里程计：

ros2 topic echo /odom --once

确认：

child_frame_id: base_link

检查 TF：

ros2 run tf2_ros tf2_echo odom base_link

确保坐标转换正常。

问题3：MuJoCo 启动后迷宫环境丢失

问题描述

部分情况下启动 MuJoCo 后只显示机器人，没有迷宫环境。

原因分析

MuJoCo 环境由 MJCF 文件定义，不同启动方式可能加载不同模型文件。

解决方法

使用正确启动方式：

ros2 launch nota_carter_mini_mujoco sim.launch.py

并确认加载的是包含迷宫的 MJCF 文件。

实验复现步骤

完整实验流程：

终端1：

启动仿真：

source ~/nota_carter_mini_ros2/install/setup.bash

ros2 launch nota_carter_mini_mujoco sim.launch.py

终端2：

启动 SLAM：

source ~/nota_carter_mini_ros2/install/setup.bash

ros2 launch nota_carter_mini_slam slam_online.launch.py

终端3：

启动 RViz：

rviz2

设置：

Fixed Frame = map

添加：

Map:

/map

LaserScan:

/scan